🌿 MQTT Mastery Project

Complete Technical Documentation

ESP32 → HiveMQ Cloud → React Web App + Flutter Android App

| Field | Details |
| --- | --- |
| Author | Radha |
| GitHub | https://github.com/radhapro/mqtt-mastery |
| Live Web App | https://react-app-xi-one-83.vercel.app |
| Date | May 2026 |
| Status | Completed ✅ |

# 1. Project Overview

MQTT Mastery ek complete end-to-end IoT (Internet of Things) project hai jisme ek ESP32 microcontroller, cloud MQTT broker, web dashboard, aur Android mobile app sab ek saath kaam karte hain. Is project ka main goal tha ki real sensor data ko wirelessly ek central server pe bheja jaye aur phir us data ko kisi bhi device pe — chahe web browser ho ya Android phone — realtime mein dekha ja sake aur hardware ko control kiya ja sake.

MQTT (Message Queuing Telemetry Transport) ek lightweight publish-subscribe messaging protocol hai jo IoT devices ke liye specifically design kiya gaya hai. Ye protocol bahut kam bandwidth use karta hai aur low power devices pe bhi aasani se run karta hai — isliye ESP32 jaise microcontrollers ke liye ye perfect choice hai.

## 1.1 Project Goals

- ESP32 ko WiFi se connect karke sensor data publish karna

- HiveMQ Cloud broker ka use karke secure MQTT communication setup karna

- React.js web dashboard banana jo realtime data show kare

- Flutter Android app banana jo phone pe data dikhaye aur LED control kare

- Poora project GitHub pe version control ke saath manage karna

- React app ko Vercel pe deploy karna

## 1.2 System Architecture

Poora system teen layers mein divide hai:

| Layer | Technology | Role |
| --- | --- | --- |
| Hardware Layer | ESP32 + Arduino | Sensor data collect karta hai aur MQTT publish karta hai |
| Cloud/Broker Layer | HiveMQ Cloud | MQTT messages route karta hai publisher se subscriber tak |
| Application Layer | React.js + Flutter | Data display karta hai aur user commands bhejta hai |

Data flow is tarah hota hai:

- ESP32 sensor data padhta hai (abhi fake data, baad mein DHT11 real sensor)

- Data JSON format mein encode karta hai

- HiveMQ Cloud pe 'home/sensor' topic pe publish karta hai

- React app aur Flutter app dono us topic ko subscribe karte hain

- Jab koi app 'home/led' topic pe ON/OFF publish kare, ESP32 us command pe act karta hai

# 2. MQTT Protocol — Deep Dive

MQTT ko samajhna is poore project ki neenv hai. Chalo isko detail mein samjhte hain.

## 2.1 MQTT Kya Hai?

MQTT ek messaging protocol hai jo 1999 mein IBM ne design kiya tha oil pipeline monitoring ke liye. Aaj ye IoT ka sabse popular protocol ban gaya hai kyunki:

- Bahut lightweight hai — HTTP se 10x kam bandwidth use karta hai

- Publish-Subscribe pattern use karta hai — sender aur receiver ko ek doosre ka IP address pata nahi hona chahiye

- Unreliable networks pe bhi kaam karta hai

- Battery powered devices ke liye perfect hai

## 2.2 Publish-Subscribe Pattern

Traditional HTTP mein client directly server se request karta hai. MQTT mein beech mein ek 'broker' hota hai:

- Publisher: Jo device data bhejta hai (ESP32) — wo ek topic pe message publish karta hai

- Broker: Central server (HiveMQ) — messages receive karta hai aur sahi subscribers tak deliver karta hai

- Subscriber: Jo device data receive karna chahta hai (React App, Flutter App) — wo topic subscribe karta hai

Example: ESP32 'home/sensor' topic pe temperature data publish karta hai. React App aur Flutter App dono ne 'home/sensor' subscribe kiya hua hai. Jab ESP32 publish karta hai, HiveMQ immediately dono apps ko wo data bhej deta hai.

## 2.3 MQTT Topics

Topics MQTT mein ek addressing system ki tarah kaam karte hain — slash (/) se hierarchical structure banate hain:

| Topic | Publisher | Subscriber | Data |
| --- | --- | --- | --- |
| home/sensor | ESP32 | React App, Flutter App | Temperature, Humidity JSON |
| home/led | React App, Flutter App | ESP32 | ON ya OFF string |

## 2.4 QoS Levels

MQTT mein teen Quality of Service levels hote hain:

- QoS 0 (At most once): Message ek baar bhejo, delivery guarantee nahi — fastest but unreliable

- QoS 1 (At least once): Message tab tak bhejo jab tak acknowledgment na mile — hamara project yahi use karta hai

- QoS 2 (Exactly once): Guarantee karo exactly ek baar delivery — slowest but most reliable

## 2.5 MQTT Ports

| Port | Protocol | Usage |
| --- | --- | --- |
| 1883 | MQTT (Plain) | Local/unsecured connections |
| 8883 | MQTT + SSL/TLS | Secure connections — ESP32 aur Flutter use karte hain |
| 8884 | MQTT over WebSocket + SSL | Browser-based connections — React App use karta hai |

# 3. HiveMQ Cloud Setup

HiveMQ world ka most popular MQTT broker hai. Hamne HiveMQ Cloud ka free tier use kiya jo fully managed hai — matlab humhe koi server setup nahi karna pada.

## 3.1 HiveMQ Account Details

| Parameter | Value |
| --- | --- |
| Broker URL | 8c77d38bb13146aeb2858d539a7cd2d0.s1.eu.hivemq.cloud |
| MQTT Port | 8883 (SSL) |
| WebSocket Port | 8884 (WSS) |
| Username | radha |
| Password | Radha@mqtt123 |
| Region | EU (Europe) |
| Plan | Free Tier |

## 3.2 HiveMQ Free Tier Limits

- Maximum 100 concurrent connections

- Maximum 10 GB data transfer per month

- SSL/TLS encryption included

- Username/password authentication

- 99.9% uptime SLA

## 3.3 Security

HiveMQ pe SSL/TLS encryption use kiya gaya hai jo ensure karta hai ki data transmit hote waqt encrypt ho. ESP32 mein 'setInsecure()' use kiya gaya hai kyunki ESP32 ke paas certificate verification ke liye enough memory nahi hoti. Production mein proper certificate use karna chahiye.

# 4. ESP32 Firmware — Complete Technical Details

ESP32 ek powerful dual-core microcontroller hai jo WiFi aur Bluetooth built-in ke saath aata hai. Ye IoT projects ke liye industry standard ban gaya hai.

## 4.1 ESP32 Specifications

| Spec | Value |
| --- | --- |
| Processor | Xtensa dual-core LX6, 240 MHz |
| RAM | 520 KB SRAM |
| Flash | 4 MB |
| WiFi | 802.11 b/g/n |
| Bluetooth | v4.2 BR/EDR + BLE |
| GPIO Pins | 34 |
| Operating Voltage | 3.3V |

## 4.2 Libraries Used

| Library | Purpose | Version |
| --- | --- | --- |
| WiFi.h | ESP32 WiFi connection | Built-in |
| WiFiClientSecure.h | SSL/TLS encrypted connection | Built-in |
| PubSubClient | MQTT client library | ^2.8 |
| ArduinoJson | JSON encode/decode | ^7.0.0 |

## 4.3 Connection Flow

ESP32 startup pe ye steps follow karta hai:

- Serial monitor initialize karta hai (115200 baud)

- LED pin ko output mode mein set karta hai (Pin 2)

- WiFi network se connect hota hai — SSID aur password use karke

- WiFiClientSecure se SSL context setup karta hai

- PubSubClient ko HiveMQ server, port, aur callback function set karta hai

- MQTT broker se connect hota hai username/password authentication ke saath

- 'home/led' topic subscribe karta hai — LED control commands ke liye

## 4.4 MQTT Topics aur Data Format

ESP32 har 5 second mein ye JSON format mein data publish karta hai:

{"temperature": 28.5, "humidity": 65.2, "device": "ESP32-Demo"}

LED control ke liye ESP32 'home/led' topic sunता hai:

- 'ON' aaya → GPIO Pin 2 HIGH → LED jal jaati hai

- 'OFF' aaya → GPIO Pin 2 LOW → LED band ho jaati hai

## 4.5 Reconnection Logic

ESP32 ka code robust hai — agar WiFi ya MQTT connection toote toh automatically reconnect karta hai. Loop function mein har iteration pe connection check hota hai aur agar disconnected ho toh reconnect attempt hota hai.

## 4.6 Abhi Fake Data Kyun?

Is phase mein ESP32 fake random data generate karta hai kyunki DHT11 sensor abhi physically connect nahi kiya gaya. random() function se 20-35 degrees ke beech temperature aur 40-80% ke beech humidity values generate hoti hain. Next step mein real DHT11 sensor lagaya jayega.

# 5. React.js Web Dashboard

React web dashboard project ka pehla working UI tha jo browser mein MQTT data realtime show karta hai.

## 5.1 Technical Stack

| Technology | Version | Purpose |
| --- | --- | --- |
| React.js | 18.x | UI framework |
| mqtt (npm) | 5.x | MQTT client for browser |
| Vite | 5.x | Build tool |
| Vercel | - | Deployment platform |

## 5.2 Browser MQTT Connection

Browser directly TCP connections nahi kar sakta — isliye WebSocket use karna padta hai. HiveMQ ka port 8884 WebSocket over SSL provide karta hai:

wss://8c77d38bb13146aeb2858d539a7cd2d0.s1.eu.hivemq.cloud:8884/mqtt

React app mein useEffect hook mein MQTT connection establish hota hai component mount hone pe. Cleanup function mein client.end() call hota hai taaki memory leaks na ho.

## 5.3 Features

- Realtime temperature aur humidity display

- Connection status indicator (Connected/Disconnected/Error)

- LED ON/OFF control buttons

- Alert system agar temperature ya humidity threshold cross kare

- Responsive design — mobile pe bhi theek dikhta hai

## 5.4 Deployment — Vercel

React app Vercel pe deploy kiya gaya jo GitHub repository se directly connected hai. Jab bhi code push hota hai GitHub pe, Vercel automatically naya build deploy kar deta hai — ye Continuous Deployment (CD) ka example hai.

- Live URL: https://react-app-xi-one-83.vercel.app

- GitHub se auto-deploy connected

- Free tier use kiya gaya

- Global CDN pe serve hota hai

# 6. Flutter Android App — Complete Technical Details

React Native try karne ke baad Flutter choose kiya gaya kyunki Flutter IoT projects ke liye better suited hai — native performance, clean MQTT library support, aur seedha APK build.

## 6.1 React Native vs Flutter — Kyun Flutter?

| Aspect | React Native | Flutter |
| --- | --- | --- |
| Language | JavaScript | Dart |
| Performance | JS Bridge se thoda slow | Native compiled — fast |
| MQTT Library | Browser-centric, issues | mqtt_client — clean aur reliable |
| APK Build | Expo issues aaye | flutter build apk — seedha kaam kiya |
| UI | Web jaisa | Truly native UI |
| IoT Support | Average | Excellent |

## 6.2 Flutter Setup

Flutter already installed tha system pe (v3.32.2). Naya project create kiya:

flutter create mqtt_app

cd mqtt_app

flutter pub add mqtt_client

## 6.3 MQTT Connection in Flutter

Flutter mein MqttServerClient use kiya gaya jo TCP over SSL support karta hai (port 8883). Browser ki tarah WebSocket ki zaroorat nahi padti — seedha TCP connection hota hai.

Key configurations:

- client.secure = true → SSL/TLS enable

- client.keepAlivePeriod = 30 → har 30 second pe heartbeat

- client.onBadCertificate = (a) => true → certificate bypass (development ke liye)

- Unique client ID → DateTime.now().millisecondsSinceEpoch se generate hota hai

## 6.4 Challenges aur Solutions

| Problem | Cause | Solution |
| --- | --- | --- |
| DNS Lookup Failed | Mobile data pe hostname resolve nahi hua | onBadCertificate = true add kiya |
| Gradle Lock Error | Previous Gradle process chal raha tha | taskkill /F /IM java.exe se Java process kill kiya |
| Internet Permission Missing | AndroidManifest.xml mein permission nahi thi | INTERNET aur ACCESS_NETWORK_STATE permission add ki |
| Expo SDK Incompatible | React Native mein SDK version mismatch | Flutter pe switch kiya — ye problem nahi aayi |
| Build Time Slow | Pehli baar Gradle dependencies download | 10-15 minute wait kiya — normal behaviour hai |

## 6.5 AndroidManifest.xml Changes

Android app ko internet access ke liye explicitly permissions declare karni hoti hain:

<uses-permission android:name="android.permission.INTERNET"/>

<uses-permission android:name="android.permission.ACCESS_NETWORK_STATE"/>

android:usesCleartextTraffic="true"

## 6.6 APK Build

Release APK build command:

flutter build apk --release

APK details:

- Size: 18.3 MB

- Type: Release (optimized, tree-shaken)

- Location: build/app/outputs/flutter-apk/app-release.apk

- Icons tree-shaken: MaterialIcons 1.6MB → 1KB (99.9% reduction)

## 6.7 App Features

- Realtime temperature display (°C)

- Realtime humidity display (%)

- MQTT connection status indicator

- LED ON button — 'home/led' topic pe 'ON' publish karta hai

- LED OFF button — 'home/led' topic pe 'OFF' publish karta hai

- Auto-reconnect on disconnect

# 7. GitHub Repository Structure

Poora project ek GitHub repository mein organized hai taaki version control, collaboration, aur portfolio showcase ho sake.

## 7.1 Repository

URL: https://github.com/radhapro/mqtt-mastery

## 7.2 Folder Structure

| Folder/File | Contents | Purpose |
| --- | --- | --- |
| esp32/ | Arduino .cpp, platformio.ini | ESP32 firmware code |
| react-app/ | React.js source, package.json | Web dashboard |
| flutter-app/ | Flutter Dart code, Android config | Android mobile app |
| flutter-app/build/.../app-release.apk | Compiled APK file | Direct download aur install |
| mqtt-android/ | React Native (experimental) | Pehla try — baad mein Flutter use kiya |
| README.md | Project documentation | Project overview aur links |
| .gitignore | Git ignore rules | node_modules etc. exclude karta hai |

## 7.3 Git Commands Used

git init

git add .

git commit -m "commit message"

git push origin main

git add -f flutter-app/build/.../app-release.apk  # Force add ignored files

# 8. Complete Tech Stack Summary

| Layer | Technology | Version | Role |
| --- | --- | --- | --- |
| Microcontroller | ESP32 | - | Hardware + WiFi |
| Firmware Language | C++ (Arduino) | - | ESP32 programming |
| MQTT Library (ESP32) | PubSubClient | ^2.8 | MQTT on ESP32 |
| JSON Library | ArduinoJson | ^7.0.0 | Data serialization |
| MQTT Broker | HiveMQ Cloud | Free Tier | Message routing |
| Web Framework | React.js | 18.x | Web dashboard |
| MQTT (Browser) | mqtt npm | 5.x | WebSocket MQTT |
| Build Tool | Vite | 5.x | React bundler |
| Deployment | Vercel | - | Web hosting |
| Mobile Framework | Flutter | 3.32.2 | Android app |
| Mobile Language | Dart | 3.8.1 | Flutter programming |
| MQTT (Mobile) | mqtt_client | - | TCP MQTT |
| Version Control | Git + GitHub | - | Code management |
| IDE | VS Code | - | Development environment |
| Package Manager | npm + pub | - | Dependencies |

# 9. Learning Outcomes

Is project ko karte waqt ye skills develop huyi:

## 9.1 Hardware Skills

- ESP32 microcontroller ke saath kaam karna

- Arduino framework mein C++ programming

- GPIO pin control (LED on/off)

- WiFi aur MQTT library integration

- Serial monitor se debugging

## 9.2 Cloud Skills

- MQTT protocol ki deep understanding

- HiveMQ Cloud broker setup aur configuration

- SSL/TLS secure connections

- Publish-Subscribe pattern implementation

## 9.3 Frontend Skills

- React.js mein realtime data display

- WebSocket based MQTT connection in browser

- Vercel pe deployment aur CI/CD

- Responsive UI design

## 9.4 Mobile Development Skills

- Flutter project setup aur structure

- Dart programming language basics

- TCP over SSL MQTT connection

- Android manifest configuration

- Release APK build aur distribution

- React Native vs Flutter comparison — practical experience

## 9.5 DevOps Skills

- Git version control — init, add, commit, push

- GitHub repository management

- APK force-add to git with -f flag

- Gradle troubleshooting

- Node.js aur npm package management

# 10. Next Steps — DHT11 Sensor Integration

Project ka agle phase mein real DHT11 temperature aur humidity sensor lagaya jayega ESP32 mein. Abhi fake random data generate ho raha hai — DHT11 se real sensor readings aayengi.

## 10.1 DHT11 Specifications

| Spec | Value |
| --- | --- |
| Temperature Range | 0°C to 50°C |
| Humidity Range | 20% to 90% RH |
| Accuracy (Temp) | ±2°C |
| Accuracy (Humidity) | ±5% RH |
| Sampling Rate | 1 reading per second |
| Voltage | 3.3V - 5V |
| Protocol | Single wire digital |

## 10.2 Wiring

| DHT11 Pin | ESP32 Pin |
| --- | --- |
| VCC | 3.3V |
| GND | GND |
| DATA | GPIO 4 (ya koi bhi digital pin) |

## 10.3 Code Changes

ESP32 code mein ye changes aayenge:

- DHT.h library add karni hogi

- dht.begin() setup mein call karein

- dht.readTemperature() aur dht.readHumidity() se real values lein

- random() calls hataani hongi

- NaN check add karna hoga (sensor kabhi kabhi NaN return karta hai)

## 10.4 Future Enhancements

- Multiple sensors add karna — soil moisture, light sensor, gas sensor

- Flutter app mein historical graph add karna (recharts ya fl_chart)

- Push notifications — agar temperature threshold cross ho

- Data storage — Firebase ya InfluxDB mein save karna

- Flutter app Vercel/Play Store pe publish karna

- Multiple ESP32 devices manage karna

— End of Document —

GitHub: https://github.com/radhapro/mqtt-mastery