MQTT Mastery Project

Complete Technical Notes with Full Source Code

| Field | Details |
| --- | --- |
| Author | Radha |
| GitHub | https://github.com/radhapro/mqtt-mastery |
| Live Web App | https://react-app-xi-one-83.vercel.app |
| HiveMQ Broker | 8c77d38bb13146aeb2858d539a7cd2d0.s1.eu.hivemq.cloud |
| MQTT Port | 8883 (SSL) ¦ 8884 (WebSocket SSL) |
| Username | radha ¦ Password: Radha@mqtt123 |
| Pub Topic | home/sensor |
| Sub Topic | home/led |

# 1. System Architecture

Poora system teen layers mein kaam karta hai:

| Layer | Technology | Role |
| --- | --- | --- |
| Hardware | ESP32 + Arduino C++ | Sensor data collect karo aur MQTT publish karo |
| Cloud Broker | HiveMQ Cloud | Messages route karo publisher se subscriber tak |
| Applications | React.js + Flutter | Data display karo aur LED control karo |

Data Flow:

- ESP32 → JSON banao → 'home/sensor' pe publish karo → HiveMQ receive karta hai

- HiveMQ → React App aur Flutter App dono ko simultaneously deliver karta hai

- React/Flutter → 'home/led' pe ON/OFF publish karo → ESP32 receive karke LED control karta hai

# 2. MQTT Protocol Samajhna

## 2.1 MQTT Kya Hai?

MQTT (Message Queuing Telemetry Transport) ek lightweight publish-subscribe protocol hai. IoT ke liye perfect hai kyunki:

- HTTP se 10x kam bandwidth use karta hai

- Publish-Subscribe pattern — sender aur receiver ko ek doosre ka IP nahi pata hona chahiye

- Unreliable networks pe bhi kaam karta hai

- Battery powered devices ke liye designed hai

## 2.2 Topics

| Topic | Publisher | Subscriber | Data |
| --- | --- | --- | --- |
| home/sensor | ESP32 | React App, Flutter App | {"temperature":28.5,"humidity":65.2} |
| home/led | React App / Flutter App | ESP32 | ON ya OFF string |

## 2.3 Ports

| Port | Protocol | Use |
| --- | --- | --- |
| 8883 | MQTT + SSL/TLS | ESP32 aur Flutter use karte hain — TCP connection |
| 8884 | MQTT + WebSocket + SSL | React.js browser use karta hai — WS connection |

# 3. ESP32 Code — main.cpp

## 3.1 Libraries (platformio.ini mein add karo)

[env:esp32dev]

platform = espressif32

board = esp32dev

framework = arduino

monitor_speed = 115200

lib_deps =

knolleary/PubSubClient@^2.8

bblanchon/ArduinoJson@^7.0.0

## 3.2 Pura ESP32 Code (esp32/src/main.cpp)

#include <Arduino.h>

#include <WiFi.h>

#include <WiFiClientSecure.h>

#include <PubSubClient.h>

#include <ArduinoJson.h>

// WiFi credentials

const char* ssid     = "Robozz Lab";

const char* password = "Robotics@cloud";

// HiveMQ credentials

const char* mqtt_server   = "8c77d38bb13146aeb2858d539a7cd2d0.s1.eu.hivemq.cloud";

const char* mqtt_username = "radha";

const char* mqtt_password = "Radha@mqtt123";

const int   mqtt_port     = 8883;

// Topics

const char* pub_topic = "home/sensor";

const char* sub_topic = "home/led";

// LED pin

const int LED_PIN = 2;

WiFiClientSecure espClient;

PubSubClient client(espClient);

// Jab dashboard se command aaye

void callback(char* topic, byte* payload, unsigned int length) {

String message = "";

for (int i = 0; i < length; i++) {

message += (char)payload[i];

}

if (message == "ON") {

digitalWrite(LED_PIN, HIGH);

} else if (message == "OFF") {

digitalWrite(LED_PIN, LOW);

}

}

void connectWiFi() {

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

}

}

void connectMQTT() {

espClient.setInsecure();

client.setServer(mqtt_server, mqtt_port);

client.setCallback(callback);

while (!client.connected()) {

String clientId = "ESP32-" + String(random(1000));

if (client.connect(clientId.c_str(), mqtt_username, mqtt_password)) {

client.subscribe(sub_topic);

} else {

delay(5000);

}

}

}

void setup() {

Serial.begin(115200);

pinMode(LED_PIN, OUTPUT);

connectWiFi();

connectMQTT();

}

void loop() {

if (!client.connected()) { connectMQTT(); }

client.loop();

// Fake sensor data (baad mein DHT11 lagayenge)

float temperature = random(20, 35) + random(0, 99) / 100.0;

float humidity    = random(40, 80) + random(0, 99) / 100.0;

JsonDocument doc;

doc["temperature"] = temperature;

doc["humidity"]    = humidity;

doc["device"]      = "ESP32-Demo";

char jsonBuffer[200];

serializeJson(doc, jsonBuffer);

client.publish(pub_topic, jsonBuffer);

delay(5000); // har 5 sec

}

## 3.3 ESP32 Code Explanation

- WiFi connect hota hai — SSID aur password se

- espClient.setInsecure() — SSL use karta hai bina certificate verify kiye

- callback() — jab 'home/led' pe message aaye toh LED on/off karo

- loop() mein har 5 second pe fake temperature/humidity JSON publish hota hai

- Reconnect logic — agar MQTT disconnect ho toh auto reconnect

## 3.4 DHT11 Lagane ke Baad Code Change

Jab real DHT11 sensor lagao tab ye changes karna:

#include "DHT.h"

#define DHTPIN 4

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

// setup() mein add karo:

dht.begin();

// loop() mein random() ki jagah:

float temperature = dht.readTemperature();

float humidity = dht.readHumidity();

if (isnan(temperature) || isnan(humidity)) { return; } // error check

# 4. Flutter Android App Code

## 4.1 Setup Commands

# Flutter project banao

flutter create mqtt_app

cd mqtt_app

# MQTT library add karo

flutter pub add mqtt_client

# APK build karo

flutter build apk --release

# APK location:

# build/app/outputs/flutter-apk/app-release.apk

## 4.2 Pura Flutter Code (lib/main.dart)

import 'package:flutter/material.dart';

import 'package:mqtt_client/mqtt_client.dart';

import 'package:mqtt_client/mqtt_server_client.dart';

import 'dart:convert';

import 'dart:io';

void main() => runApp(const MyApp());

class MyApp extends StatelessWidget {

const MyApp({super.key});

@override

Widget build(BuildContext context) {

return MaterialApp(

title: 'ESP32 Monitor',

theme: ThemeData.dark(),

home: const MqttScreen(),

);

}

}

class MqttScreen extends StatefulWidget {

const MqttScreen({super.key});

@override

State<MqttScreen> createState() => _MqttScreenState();

}

class _MqttScreenState extends State<MqttScreen> {

String temperature = '--';

String humidity = '--';

String status = 'Connecting...';

MqttServerClient? client;

@override

void initState() {

super.initState();

connectMqtt();

}

Future<void> connectMqtt() async {

final client = MqttServerClient.withPort(

'8c77d38bb13146aeb2858d539a7cd2d0.s1.eu.hivemq.cloud',

'flutter_client_${DateTime.now().millisecondsSinceEpoch}',

8883,

);

client.secure = true;

client.logging(on: false);

client.keepAlivePeriod = 30;

client.onBadCertificate = (dynamic a) => true;

client.onDisconnected = () => setState(() => status = "Disconnected");

final SecurityContext context = SecurityContext.defaultContext;

client.securityContext = context;

final connMessage = MqttConnectMessage()

.withClientIdentifier('flutter_client_${DateTime.now().millisecondsSinceEpoch}')

.authenticateAs('radha', 'Radha@mqtt123')

.startClean();

client.connectionMessage = connMessage;

try {

await client.connect();

this.client = client;

setState(() => status = 'Connected');

client.subscribe('home/sensor', MqttQos.atLeastOnce);

client.updates!.listen((messages) {

final message = messages[0].payload as MqttPublishMessage;

final payload = MqttPublishPayload.bytesToStringAsString(

message.payload.message);

final doc = jsonDecode(payload);

setState(() {

temperature = doc['temperature'].toStringAsFixed(1);

humidity = doc['humidity'].toStringAsFixed(1);

});

});

} catch (e) {

setState(() => status = 'Error: $e');

}

}

void publishMessage(String topic, String message) {

if (client == null) return;

final builder = MqttClientPayloadBuilder();

builder.addString(message);

client!.publishMessage(topic, MqttQos.atLeastOnce, builder.payload!);

}

@override

Widget build(BuildContext context) {

return Scaffold(

backgroundColor: const Color(0xFF0F172A),

body: Center(

child: Column(

mainAxisAlignment: MainAxisAlignment.center,

children: [

const Text('ESP32 Monitor',

style: TextStyle(fontSize: 28, color: Colors.white, fontWeight: FontWeight.bold)),

const SizedBox(height: 8),

Text(status, style: const TextStyle(color: Colors.grey)),

const SizedBox(height: 32),

_card('Temperature', '$temperature C'),

const SizedBox(height: 16),

_card('Humidity', '$humidity %'),

const SizedBox(height: 32),

Row(

mainAxisAlignment: MainAxisAlignment.center,

children: [

ElevatedButton(

style: ElevatedButton.styleFrom(backgroundColor: Colors.green),

onPressed: () => publishMessage('home/led', 'ON'),

child: const Text('ON'),

),

const SizedBox(width: 24),

ElevatedButton(

style: ElevatedButton.styleFrom(backgroundColor: Colors.red),

onPressed: () => publishMessage('home/led', 'OFF'),

child: const Text('OFF'),

),

],

),

],

),

),

);

}

Widget _card(String label, String value) {

return Container(

width: 280,

padding: const EdgeInsets.all(24),

decoration: BoxDecoration(

color: const Color(0xFF1E293B),

borderRadius: BorderRadius.circular(16),

),

child: Column(

children: [

Text(label, style: const TextStyle(color: Colors.grey, fontSize: 16)),

const SizedBox(height: 8),

Text(value, style: const TextStyle(color: Color(0xFF38BDF8), fontSize: 48, fontWeight: FontWeight.bold)),

],

),

);

}

}

## 4.3 AndroidManifest.xml (android/app/src/main/AndroidManifest.xml)

INTERNET permission zaroor add karo — bina iske app internet use nahi kar sakti:

<manifest xmlns:android="http://schemas.android.com/apk/res/android">

<uses-permission android:name="android.permission.INTERNET"/>

<uses-permission android:name="android.permission.ACCESS_NETWORK_STATE"/>

<application

android:label="mqtt_app"

android:name="${applicationName}"

android:icon="@mipmap/ic_launcher"

android:usesCleartextTraffic="true">

<activity

android:name=".MainActivity"

android:exported="true"

android:launchMode="singleTop"

android:theme="@style/LaunchTheme">

<intent-filter>

<action android:name="android.intent.action.MAIN"/>

<category android:name="android.intent.category.LAUNCHER"/>

</intent-filter>

</activity>

<meta-data android:name="flutterEmbedding" android:value="2" />

</application>

</manifest>

## 4.4 Common Errors aur Fix

| Error | Fix |
| --- | --- |
| DNS Lookup Failed / SocketException | client.onBadCertificate = (dynamic a) => true; add karo |
| Gradle Lock Error | Terminal mein: taskkill /F /IM java.exe — phir dobara build karo |
| INTERNET Permission Denied | AndroidManifest.xml mein INTERNET permission add karo |
| Project incompatible with Expo Go | Flutter use karo — React Native chhodo |
| Build bahut slow hai | Normal hai pehli baar — Gradle download karta hai — wait karo |

# 5. React.js Web Dashboard Code

## 5.1 Setup Commands

# React project banao (mqtt-mastery folder ke andar)

npm create vite@latest react-app -- --template react

cd react-app

npm install

npm install mqtt

# Local test

npm run dev

# Vercel deploy

npm install -g vercel

vercel --prod

## 5.2 Pura React Code (src/App.jsx)

import { useEffect, useState } from 'react';

import mqtt from 'mqtt';

const MQTT_CONFIG = {

host: 'wss://8c77d38bb13146aeb2858d539a7cd2d0.s1.eu.hivemq.cloud:8884/mqtt',

username: 'radha',

password: 'Radha@mqtt123',

};

export default function App() {

const [temperature, setTemperature] = useState('--');

const [humidity, setHumidity] = useState('--');

const [status, setStatus] = useState('Connecting...');

const [client, setClient] = useState(null);

useEffect(() => {

const mqttClient = mqtt.connect(MQTT_CONFIG.host, {

username: MQTT_CONFIG.username,

password: MQTT_CONFIG.password,

});

mqttClient.on('connect', () => {

setStatus('Connected');

mqttClient.subscribe('home/sensor');

setClient(mqttClient);

});

mqttClient.on('message', (topic, message) => {

const data = JSON.parse(message.toString());

setTemperature(data.temperature.toFixed(1));

setHumidity(data.humidity.toFixed(1));

});

mqttClient.on('error', () => setStatus('Error'));

return () => mqttClient.end();

}, []);

const sendCommand = (cmd) => {

if (client) client.publish('home/led', cmd);

};

return (

<div style={{ background: "#0f172a", minHeight: "100vh", display: "flex",

flexDirection: "column", alignItems: "center", justifyContent: "center" }}>

<h1 style={{ color: "white" }}>ESP32 Monitor</h1>

<p style={{ color: "gray" }}>{status}</p>

<div style={{ background: "#1e293b", borderRadius: 16, padding: 24, margin: 8 }}>

<p style={{ color: "gray" }}>Temperature</p>

<h2 style={{ color: "#38bdf8", fontSize: 48 }}>{temperature}°C</h2>

</div>

<div style={{ background: "#1e293b", borderRadius: 16, padding: 24, margin: 8 }}>

<p style={{ color: "gray" }}>Humidity</p>

<h2 style={{ color: "#38bdf8", fontSize: 48 }}>{humidity}%</h2>

</div>

<div style={{ display: "flex", gap: 16, marginTop: 24 }}>

<button onClick={() => sendCommand("ON")}

style={{ background: "green", color: "white", padding: "12px 32px", borderRadius: 8, border: "none", fontSize: 18 }}>

ON

</button>

<button onClick={() => sendCommand("OFF")}

style={{ background: "red", color: "white", padding: "12px 32px", borderRadius: 8, border: "none", fontSize: 18 }}>

OFF

</button>

</div>

</div>

);

}

## 5.3 React App Points

- Browser mein TCP nahi hota — isliye WebSocket (wss://) use kiya port 8884 pe

- useEffect mein MQTT connect hota hai component mount pe

- Cleanup function mein client.end() — memory leak se bachao

- JSON.parse() se temperature aur humidity nikalo

- client.publish() se LED command bhejo

# 6. GitHub Setup

## 6.1 Repository Structure

mqtt-mastery/

├── esp32/

│   └── src/main.cpp          ← ESP32 firmware

├── react-app/

│   ├── src/App.jsx           ← React dashboard

│   └── package.json

├── flutter-app/

│   ├── lib/main.dart         ← Flutter app code

│   ├── android/app/src/main/AndroidManifest.xml

│   └── build/.../app-release.apk  ← Compiled APK

├── README.md

└── .gitignore

## 6.2 Git Commands

# Pehli baar setup

git init

git remote add origin https://github.com/radhapro/mqtt-mastery.git

# Roz ka kaam

git add .

git commit -m "kya change kiya"

git push origin main

# APK force add (gitignore bypass)

git add -f flutter-app/build/app/outputs/flutter-apk/app-release.apk

git commit -m "APK added"

git push origin main

# 7. Complete Tech Stack

| Technology | Version | Purpose | Install Command |
| --- | --- | --- | --- |
| Node.js | v22.12.0 | JavaScript runtime | nodejs.org se download |
| Flutter | 3.32.2 | Mobile app framework | flutter.dev se download |
| Dart | 3.8.1 | Flutter ki language | Flutter ke saath aata hai |
| React.js | 18.x | Web UI framework | npm create vite@latest |
| mqtt (npm) | 5.x | Browser MQTT client | npm install mqtt |
| mqtt_client (pub) | - | Flutter MQTT client | flutter pub add mqtt_client |
| PubSubClient | ^2.8 | ESP32 MQTT library | platformio.ini mein add |
| ArduinoJson | ^7.0.0 | JSON for ESP32 | platformio.ini mein add |
| HiveMQ Cloud | Free | MQTT Broker | hivemq.com pe account banao |
| Vercel | Free | Web hosting | vercel.com pe account banao |
| Git | - | Version control | git-scm.com se download |
| VS Code | - | Code editor | code.visualstudio.com |

# 8. Dobara Banana Ho To — Step by Step Guide

Is guide ko follow karke exactly same project dobara bana sakti ho:

## Step 1 — Tools Install Karo

- Node.js install karo — nodejs.org

- Flutter install karo — flutter.dev

- VS Code install karo — code.visualstudio.com

- Git install karo — git-scm.com

- HiveMQ Cloud account banao — hivemq.com (free)

- Vercel account banao — vercel.com (free)

## Step 2 — GitHub Repository Banao

mkdir mqtt-mastery

cd mqtt-mastery

git init

git remote add origin https://github.com/TUMHARA_USERNAME/mqtt-mastery.git

## Step 3 — ESP32 Setup

- VS Code mein PlatformIO extension install karo

- New PlatformIO project banao — board: ESP32 Dev Module

- platformio.ini mein libraries add karo (Section 3.1 dekho)

- main.cpp mein Section 3.2 ka code paste karo

- WiFi credentials aur HiveMQ credentials update karo

- Upload karo ESP32 pe

## Step 4 — React App Banao

npm create vite@latest react-app -- --template react

cd react-app

npm install mqtt

- src/App.jsx mein Section 5.2 ka code paste karo

- npm run dev se test karo

- vercel --prod se deploy karo

## Step 5 — Flutter App Banao

flutter create mqtt_app

cd mqtt_app

flutter pub add mqtt_client

- lib/main.dart mein Section 4.2 ka code paste karo

- AndroidManifest.xml mein Section 4.3 ka code paste karo

- flutter build apk --release se APK banao

- APK phone pe install karo

## Step 6 — Sab GitHub Pe Push Karo

cd mqtt-mastery

git add .

git commit -m "Initial commit — full MQTT project"

git push origin main

Project Complete! Ab DHT11 real sensor lagao aur data live dekho!

GitHub: https://github.com/radhapro/mqtt-mastery